WINDCON

— Leader of Motor Control —

Permanent Magnet Synchronous Motor Controller

USER MANUAL

Model: FDK3533C-XC  (Electric Tricycle Version)

Figure 1 — FDK3533C-XC annotated controller — all major components labelled

Document Information Version: V1.0 EN ¦ Based on Oct 2023 original ¦ For internal / NDA use only Guilin Windcon Technology Co., Ltd. ¦ www.windcon.com.cn

# Table of Contents

| Preface & Version History | 3 |
| --- | --- |

| 1 Safety Information | 3 |
| --- | --- |

| 2 Product Overview | 5 |
| --- | --- |

| 3 Controller Port Description | 6 |
| --- | --- |

| 3.1 Physical Layout | 6 |
| --- | --- |

| 3.2 30-Pin IO Connector | 7 |
| --- | --- |

| 3.3 8-Pin Encoder Connector | 9 |
| --- | --- |

| 3.4 Power & Motor Terminal Studs | 10 |
| --- | --- |

| 4 Controller Resource Configuration | 10 |
| --- | --- |

| 4.1 Signal Inputs | 10 |
| --- | --- |

| 4.2 Signal Output | 11 |
| --- | --- |

| 4.3 Voltage Outputs | 11 |
| --- | --- |

| 4.4 Communication Interfaces | 11 |
| --- | --- |

| 4.5 Encoder & Temperature Feedback | 12 |
| --- | --- |

| 4.6 Original Wiring Diagrams | 12 |
| --- | --- |

| 5 Installation | 14 |
| --- | --- |

| 5.1 Environmental Requirements | 14 |
| --- | --- |

| 5.2 Mechanical Dimensions | 14 |
| --- | --- |

| 5.3 Electrical Installation Safety | 15 |
| --- | --- |

| 5.4 Post-Installation Checklist | 15 |
| --- | --- |

| 6 Commissioning | 16 |
| --- | --- |

| 7 Typical Problems & Fault Analysis | 17 |
| --- | --- |

| 8 Fault Alarms & Remedies | 17 |
| --- | --- |

| Appendix A — List of Figures | 19 |
| --- | --- |

| Appendix B — List of Abbreviations | 20 |
| --- | --- |

| Appendix C — Technical Explainer Diagrams | 22 |
| --- | --- |

| C.1 Resolver Operating Principle | 22 |
| --- | --- |

| C.2 FOC / SVPWM Control Loop | 22 |
| --- | --- |

| C.3 Signal Logic Guide | 23 |
| --- | --- |

| C.4 Fault Beep Code Chart | 23 |
| --- | --- |

| C.5 Integration Guide | 24 |
| --- | --- |

| C.6 Commissioning Flowchart | 25 |
| --- | --- |

# Preface

Thank you for your support of Windcon FDK series motor controllers. This manual covers FDK3533C-XC safety, mechanical and electrical installation, port descriptions, commissioning, maintenance, and integration guidance with fully annotated diagrams.

Read carefully before use and retain for future reference. Contact Windcon technical support for assistance.

## Version History

| Date | Version | Description |
| --- | --- | --- |
| October 2023 | V1.0 | Initial Chinese release |
| (current) | V1.0 EN | English translation + all annotated diagrams + appendices + explainer sections |

# 1  Safety Information

## 1.1  Safety Alert Levels

⚠ DANGER — Death or serious injury WILL result if instructions are not followed.

⚠ WARNING — Death or serious injury MAY result if instructions are not followed.

⚠ CAUTION — Minor injury or equipment damage may result if instructions are not followed.

## 1.2  General Safety Declarations

- Read and follow all safety information before installing, operating, or maintaining this product.

- Safety warnings in this manual are supplementary and do not represent all possible hazards.

- Use only in environments meeting design specifications. Non-compliance voids warranty.

Windcon assumes no liability for damage caused by:

- Non-compliance with operating instructions

- Unauthorized modification or use in special applications (e.g. lifting equipment)

- Improper operation

- Natural events such as flooding or lightning

- Damage incurred during transportation

## 1.3  Safety Precautions by Phase

### Wiring

⚠ DANGER — Wiring

- Non-professionals STRICTLY PROHIBITED from wiring! - Never wire while power is connected — risk of electric shock. - Cut ALL power and wait at least 15 minutes before touching — capacitor residual voltage remains. - Ensure reliable grounding of all equipment — risk of electric shock if omitted. - Follow ESD procedures; wear anti-static wristband when wiring.

⚠ WARNING — Wiring

- NEVER connect power supply to motor output terminals U/V/W — causes damage and may cause fire! - Ensure controller and motor terminal phase sequence match exactly to avoid stall or runaway. - Cables must meet required cross-section and shielding. Shielded cable shield: single-end ground only. - After wiring: verify polarity carefully — reversed polarity damages product or motor encoder.

### Power-On

⚠ DANGER — Power-On

- Motor MUST be unloaded for all initial commissioning — required for safety and correct zeroing. - Confirm supply voltage matches controller rating before power-on. - At power-on, mechanical devices may suddenly move — maintain sufficient safe distance! - Never touch wiring terminals, open covers, or disassemble anything while energized.

### During Operation

⚠ DANGER — During Operation

- Never touch any terminal, open any cover, or disassemble anything while running. - Never touch housing, fan, motor, or resistors to test temperature — severe burn risk! - Non-professionals must not probe signals during operation — risk of personal injury.

⚠ WARNING — During Operation

- Prevent foreign objects or metal items from falling into the equipment during operation. - Do NOT use a contactor on/off method to control start/stop — causes equipment damage.

### Maintenance & Repair

⚠ DANGER — Maintenance / Repair

- Non-professionals STRICTLY PROHIBITED from maintenance or repair! - Never work on powered equipment. Cut all power and wait at least 15 minutes. - After replacement: re-confirm all wiring, correct all parameters, run motor unloaded first. - Do not continue using a damaged product — greater damage or accident will result.

# 2  Product Overview

## 2.1  Product Introduction

The FDK3533C-XC is a low-voltage Permanent Magnet Synchronous Motor (PMSM) controller designed for electric tricycles, developed by Guilin Windcon Technology Co., Ltd. It employs advanced Field-Oriented Control (FOC / SVPWM) algorithms for precise torque control across a wide speed range, delivering stable and smooth driving. Multiple industry-leading safety technologies ensure reliability under all operating conditions.

## 2.2  Key Features

- Resolver encoder: High-temp resistant, non-demagnetizing, vibration/dust/oil-proof. No hall sensors. No damage from overheating or water ingress.

- Dual thermal protection: Built-in temperature sensors in motor and controller with overheat algorithms. Reduces failure and repair rates significantly.

- Anti-rollback (hill-hold), downhill speed limiting, regenerative braking.

- Flux weakening + 3-speed mode: Adds up to 8 km/h to top speed.

- Speed: 40–55 km/h  |  Load: 1 ton  |  Max gradient: 30%  |  Efficiency: up to 94%

- Soft-start, anti-gear-clash algorithms  |  CAN + RS485 communication

## 2.3  Key Technical Specifications

| Parameter | Specification |
| --- | --- |
| Model | FDK3533C-XC |
| Dimensions (without rain cover) | 205 × 165 × 71.5 mm |
| Dimensions (with rain cover) | 205 × 176.6 × 82.1 mm |
| Max Bus Current | DC 140 A |
| Max Phase Current | 350 A rms |
| Rated Operating Voltage | 48 / 60 / 72 V DC |
| Supply Voltage Range | 18 – 96 V DC |
| Control Method | SVPWM (Space Vector PWM) |
| Insulation Resistance | >=20 MΩ (normal) ¦ >=1 MΩ (high temp/humidity) |
| Protection Rating | IP65 |
| Cooling | Natural convection — install in ventilated location |
| Motor Type | Permanent Magnet Synchronous Motor (PMSM) |
| Encoder Type | Resolver (rotary transformer) |
| Operating Temperature | -25°C to +60°C |
| Storage Temperature | -40°C to +85°C |
| Relative Humidity | 0–100% RH, non-condensing |

### Supported Functions

| Function | ✓ | Function | ✓ |
| --- | --- | --- | --- |
| Anti-runaway protection | ✓ | Anti-theft motor lock | ✓ |
| Throttle fault protection | ✓ | READY signal output | ✓ |
| Encoder fault protection | ✓ | P-gear (parking) support | ✓ |
| Driver overheat protection | ✓ | High / Low brake | ✓ |
| Motor overheat protection | ✓ | CAN communication | ✓ |
| Low-speed hill descent | ✓ | RS485 communication | ✓ |
| Hill speed limiting | ✓ | Pulse speedometer output | ✓ |
| Anti-rollback (hill hold) | ✓ | Single-wire instrument | ✓ |
| Flux weakening speed boost | ✓ | 3-speed selector | ✓ |
| Regenerative braking | ✓ | Speed loop control | ✓ |
| Soft start | ✓ | Motor direction switching | ✓ |

# 3  Controller Port Description

## 3.1  Physical Layout

The FDK3533C-XC has four main connection areas visible from the top. See annotated photo (Figure 1) and original technical drawing below.

Figure 2 — Controller port distribution — original technical drawing with English labels added

| Area | Connector / Terminals | Contents |
| --- | --- | --- |
| Power & Motor | 5× stud terminals (B+, B−, U, V, W) | Battery +/− and motor phases (25–36 mm² cable) |
| Functional IO | 30-pin connector (Connector A) | Signals, CAN, RS485, throttle, brakes, key switch, speed, READY |
| Encoder Feedback | 8-pin connector (Connector B) | Resolver SIN/COS/REF (±) and motor temperature sensor |
| Mechanical Mounting | 4× mounting holes | Diameter Ø8.5 mm, M8 bolts, 185×145 mm bolt pattern |

## 3.2  30-Pin IO Connector (Connector A)

Connector type: Electric vehicle controller 30P harness connector plug. Wire colors listed are for reference — actual harness colors may vary.

Figure 3 — 30-Pin IO connector — original layout schematic with English labels

Figure 4 — 30-Pin IO connector — color-coded pin map by signal type

| PIN | Wire Color | Signal Name | Description |
| --- | --- | --- | --- |
| A1 | Brn/Wht | Wheel Motion | Pulse output when motor rotates — connect to alarm/buzzer |
| A2 | Red/Wht | CAN Comm. L | CAN bus Low (protocol customizable per application) |
| A3 | Red | CAN Comm. H | CAN bus High (protocol customizable per application) |
| A4 | LGrn/Wht | Isolated GND | CAN/RS485 isolation ground — connect to chassis in noisy environments |
| A5 | Pnk/Wht | Isolated 5V | Isolated 5V output (max 100 mA) — reserved for Bluetooth module |
| A6 | Grn/Wht | Speed Pulse / 1-Wire | 8 pulses/rev (5V) for speedometer, OR single-wire instrument comm. |
| A7 | Black | Instrument GND | Ground wire for pulse-type speedometers |
| A8 | Blu/Wht | RS485 T+ | RS485 transmit positive — commissioning or command control |
| A9 | Pur/Wht | RS485 T− | RS485 transmit negative — commissioning or command control |
| A10 | Purple | Key Switch | Control circuit power supply (battery B+ input via key switch) |
| A11 | Brown | Reverse | Active LOW → reverse motor direction |
| A12 | Orange | Mid/Low Speed | Active LOW → switch to mid/low speed mode |
| A13 | Purple | Key Switch (Alarm) | Alarm unit sends key-on signal to start controller |
| A14 | Lt Blue | Alarm / Lock | Active LOW → alarm active, motor locked (anti-theft) |
| A15 | Blue | High Speed | Active LOW → high speed mode with flux weakening |
| A16 | Black | Signal GND | Common GND for all active-low signal inputs |
| A17 | Gray | Low Brake | Active LOW → brake engage (typically brake lever switch) |
| A18 | Yellow | P-Gear | Active LOW → enter/exit parking gear mode |
| A19 | Black | Signal GND | Common GND for active-low signals |
| A20 | Org/Wht | Voltage Selection | Active LOW → switch battery voltage profile (48/60/72V) |
| A21 | Yel/Blk | P-Gear Select | Active LOW → enable or disable P-gear feature |
| A22 | Pink | Throttle Supply + | 5V output to power throttle sensor (max 200 mA) |
| A23 | Black | Direction Select | Active LOW → swap motor phase lines to reverse direction |
| A24 | Gry/Wht | High Brake | Active HIGH (12V) → brake engage — use A25 as 12V source |
| A25 | Lt Green | 12V+ Output | 12V output for active-high switch signals only (max 200 mA) |
| A26 | White | READY Signal | 5V output to light READY lamp on instrument cluster |
| A27 | Blk/Wht | Signal GND | Common GND for active-low signals |
| A28 | Yel/Wht | B+ Output | Battery voltage output for key switch signals only (max 1 A) |
| A29 | Black | Throttle GND | Ground for throttle power supply circuit |
| A30 | Green | Throttle Signal | Throttle analog input 0–5V (from throttle/accelerator sensor) |

⚠ CAUTION — 30-Pin Connector

- Connector has a specific insertion direction — NEVER insert reversed, this WILL cause damage! - Wire harness spec: 0.5 mm² ROHS, pull force ≥6 kgf, IP66 housing, rated 80°C 300V.

## 3.3  8-Pin Encoder Feedback Connector (Connector B)

Carries all resolver signals plus motor temperature. Always use shielded cable. Ground shield at controller end only.

Figure 5 — 8-Pin encoder connector — original layout schematic with English labels

Figure 6 — 8-Pin encoder connector — color-coded SIN/COS/REF and motor temperature diagram

| PIN | Wire Color | Signal Name | Description |
| --- | --- | --- | --- |
| B1 | Red/Wht | Motor Temp (+) | Motor temperature sensor positive — default KTY84-150 |
| B2 | Blue | COS+ | Resolver cosine signal positive |
| B3 | Yellow | SIN+ | Resolver sine signal positive |
| B4 | Red | REF+ | Resolver excitation/reference signal positive (~10 kHz AC) |
| B5 | Blk/Wht | Motor Temp (−) | Motor temperature sensor negative — default KTY84-150 |
| B6 | White | COS− | Resolver cosine signal negative |
| B7 | Green | SIN− | Resolver sine signal negative |
| B8 | Black | REF− | Resolver excitation/reference signal negative |

⚠ CAUTION — 8-Pin Encoder Connector

- Connector has a specific insertion direction — check carefully; reverse insertion is PROHIBITED! - Non-KTY84-150 temperature sensors require Windcon engineering adaptation before use. - Use shielded cable for all resolver signals. Ground shield at controller end only.

## 3.4  Power & Motor Terminal Studs

Figure 7 — Power and motor terminal layout — original technical drawing with English labels

| Position | Label | Function | Cable Size | Terminal | Crimp Tool |
| --- | --- | --- | --- | --- | --- |
| 1 (leftmost) | U | Motor Phase U | 25–36 mm² | RNB 38-6 | ZTGYQK-70 |
| 2 | B− | Battery Negative | 25–36 mm² | RNB 38-6 | ZTGYQK-70 |
| 3 | V | Motor Phase V | 25–36 mm² | RNB 38-6 | ZTGYQK-70 |
| 4 | B+ | Battery Positive | 25–36 mm² | RNB 38-6 | ZTGYQK-70 |
| 5 (rightmost) | W | Motor Phase W | 25–36 mm² | RNB 38-6 | ZTGYQK-70 |

⚠ WARNING — Power Terminals

- NEVER reverse Battery B+ and B− connections! - Recommended wiring order left→right: U → B− → V → B+ → W - Cables must be 25–36 mm² tinned multi-strand copper. Crimp only with ZTGYQK-70 tool.

# 4  Controller Resource Configuration

## 4.1  Signal Inputs (11 total)

| PIN | Signal Name | Trigger Type | Input Range | Max Voltage |
| --- | --- | --- | --- | --- |
| A11 | Reverse | Active Low (0V typ.) | 0–0.2 V | 24V |
| A12 | Mid/Low speed | Active Low (0V typ.) | 0–0.2 V | 24V |
| A14 | Alarm / Motor lock | Active Low (0V typ.) | 0–0.2 V | 24V |
| A15 | High speed | Active Low (0V typ.) | 0–0.2 V | 24V |
| A17 | Low brake | Active Low (0V typ.) | 0–0.2 V | 24V |
| A18 | P-gear | Active Low (0V typ.) | 0–0.2 V | 24V |
| A20 | Voltage selection | Active Low (0V typ.) | 0–0.2 V | 24V |
| A21 | P-gear select | Active Low (0V typ.) | 0–0.2 V | 24V |
| A23 | Direction select | Active Low (0V typ.) | 0–0.2 V | 24V |
| A24 | High brake | Active High (12V typ.) | 10–18 V | 24V |
| A30 | Throttle (analog) | 0–5V nominal | 0–10 V | 12V |

⚠ CAUTION — Signal Inputs

- All signals must operate within their typical voltage range. Exceeding limits may damage the controller. - Inputs are positive-voltage triggered. Reference ground is B−. Never inject negative voltage.

## 4.2  Signal Output

| PIN | Signal | Output Voltage | Max Current |
| --- | --- | --- | --- |
| A26 | READY | 5V ±2% | 50 mA |

## 4.3  Voltage Outputs (4 total)

| PIN | Signal | Voltage | Max Current | Permitted Use |
| --- | --- | --- | --- | --- |
| A5 | Isolated 5V | 5V ±2% | 100 mA | Bluetooth module only |
| A22 | Throttle supply + | 5V ±2% | 200 mA | Throttle or hall sensor power |
| A25 | 12V+ output | 12V ±2% | 200 mA | Active-high (12V) switch signals only |
| A28 | B+ output | Battery voltage | 1 A | Key switch signal only |

⚠ CAUTION — Voltage Outputs

- These outputs may ONLY power: throttle, Bluetooth, linear brake, key switch, active-high IO. - Do NOT use to power vehicle lights, instruments, or other loads — overcurrent damages controller. - Never inject external voltage into any output pin.

## 4.4  Communication Interfaces

| Interface | Pins | Type | Key Notes |
| --- | --- | --- | --- |
| CAN Bus | A2 (CAN L), A3 (CAN H) | Isolated CAN | Built-in 120Ω. Add external 120Ω at both bus ends. Protocol customized by Windcon. |
| CAN Isolation GND | A4 (ISO_GND) | Isolation GND | Connect to chassis ground in high-interference environments. |
| RS485 | A8 (T+), A9 (T−) | Isolated RS485 | Built-in 120Ω. For commissioning and command-mode control. |
| Pulse Speedometer | A6, A7 (GND) | 5V pulse output | 8 pulses/revolution (5V). A7 = instrument GND. Default factory mode for A6. |
| Single-Wire Comm. | A6 only | Single-wire | Firmware-selected mode. Protocol requires Windcon customization. A7 not used. |
| CAN Instrument | A2, A3 | Isolated CAN | Shared CAN bus with BMS/VCU. Protocol customized separately. |

## 4.5  Encoder and Temperature Feedback

| Pins | Signal | Type | Notes |
| --- | --- | --- | --- |
| B2 (COS+), B6 (COS−) | Cosine signal | Resolver differential | Connect to motor resolver coil wires |
| B3 (SIN+), B7 (SIN−) | Sine signal | Resolver differential | Connect to motor resolver coil wires |
| B4 (REF+), B8 (REF−) | Excitation / REF | ~10 kHz AC | Supplied by controller DSP to resolver primary winding |
| B1, B5 | Motor temperature | KTY84-150 (default) | Other sensor types require Windcon engineering adaptation |

## 4.6  Original Wiring Diagrams

The following two diagrams are the original factory wiring schematics translated to English. They show how all 30 IO pins and all 8 encoder pins connect in a complete vehicle installation.

Figure 9 — Typical application wiring diagram — Variant A (single DC-DC, 12V only)

Figure 10 — Typical application wiring diagram — Variant B (DC-DC with 12V+ and 12V−)

Figure 11 — System wiring overview — all external connections with color-coded signal paths

# 5  Installation

## 5.1  Environmental Requirements

- Do NOT install in corrosive gas/liquid, flammable/explosive, water/steam/oil/dust, or high-heat environments

- Do NOT install where severe electromagnetic interference or severe grid fluctuations are present

- For vibration environments: contact Windcon technical support before installation

## 5.2  Mechanical Dimensions and Mounting

Figure 8 — Mechanical installation dimensions — original factory drawing

| Dimension | Value |
| --- | --- |
| Overall length | 205.0 mm |
| Width (without rain cover) | 165.0 mm |
| Width (with rain cover) | 176.6 mm |
| Height (without rain cover) | 71.5 mm |
| Height (with rain cover) | 82.1 mm |
| Mounting hole span (length) | 185.0 mm (centre-to-centre) |
| Mounting hole span (width) | 145.0 mm (centre-to-centre) |
| Mounting hole diameter | 4× Ø8.5 mm — M8 bolts recommended |
| Base-to-datum offset | 22.0 mm |

## 5.3  Electrical Installation Safety

- All external wiring must be performed by trained, authorized engineers only.

- Wiring must follow the wiring diagrams and this manual exactly.

- Check controller voltage rating before powering on — mismatch causes damage or fire.

- During energized servo state, no personnel may enter the mechanical movement zone.

- Cut ALL system power and wait at least 10 minutes before opening the controller housing.

- The controller has been factory dielectric-tested — do NOT repeat dielectric strength tests.

- NEVER connect the power supply to motor output terminals U, V, or W.

- Both controller and motor must be reliably grounded.

- Connect controller to supply via circuit breaker or fuse for input overcurrent protection.

## 5.4  Post-Installation Inspection Checklist

| Inspection Item | Pass Criteria |
| --- | --- |
| Controller physical condition | No damage; securely mounted; heatsink fins unobstructed |
| Cable condition | No damage/breaks; power cables separate from signal and encoder cables |
| Connector mating | All connectors fully mated; no mechanical tension; sufficient cable slack |
| Power polarity | B+ and B− correct; power NOT connected to signal or encoder ports |
| Motor phase sequence | U, V, W correct sequence; no stripped insulation on power cables |
| Ground connections | Power and ground cables correctly and firmly connected; equipment grounded |
| Terminal voltages | All terminals within typical supply range; no over-voltage |
| Encoder wiring | Resolver lines correctly matched; shielded cable used; shield grounded at one end |

# 6  Commissioning

Commissioning software: 'Windcon Debug Assistant'. Refer to its user manual for detailed usage. If a version mismatch error appears at startup, a startup configuration file is missing — contact Windcon to obtain it.

## 6.1  Commissioning Sequence

| Step | Action | Critical Notes |
| --- | --- | --- |
| 1 | Mount controller in a ventilated location | Heatsink must have clearance for airflow. Do not obstruct fins. |
| 2 | Wire B+, B−, U, V, W power terminals | Verify polarity. Follow recommended phase order. Crimp properly. |
| 3 | Wire resolver encoder B2–B8 and motor temp B1/B5 | Use shielded cable. Check 8-pin connector orientation before mating. |
| 4 | Wire all 30-pin IO signals | Key switch, throttle (A22/A29/A30), brakes, direction, speed selector. |
| 5 | Set motor to NO LOAD — no mechanical load attached | REQUIRED for all initial commissioning and electrical angle zeroing. |
| 6 | Connect PC to RS485 via USB adapter | Open Windcon Debug Assistant software on PC. |
| 7 | Power ON the controller | Listen for alarm beep codes (see Section 8). Silence = normal startup. |
| 8 | Verify PC communication with controller | Read all parameters; confirm voltage setting matches battery pack. |
| 9 | Run current loop test | Feedback RPM MUST be POSITIVE. If negative → change Swap Phase Lines (0↔1). |
| 10 | Perform electrical angle zeroing | Motor must be unloaded. Repeat 3+ times. Offset error must be <±50. |
| 11 | Test throttle response | Slow ramp-up; verify smooth speed control, no oscillation or hunting. |
| 12 | Test brakes, direction, and speed selector | All safety and control inputs must function before load testing. |
| 13 | Gradually test under increasing load | Monitor motor and controller temperatures during extended runs. |

## 6.2  Key Parameters to Verify

| Parameter | Function | What to Check / Set |
| --- | --- | --- |
| Motor rated current | Sets current limit protecting motor | Must not exceed controller rated value; confirm with motor nameplate |
| Motor pole pairs | Used in RPM formula: RPM = freq×60÷pole pairs | Incorrect value causes wrong speed display and control instability |
| Swap motor phase lines (0/1) | Reverses motor rotation direction | Set so feedback speed is POSITIVE in current loop test |
| Electrical angle zero offset | Sets rotor position reference for FOC | Must be within ±50 after repeated zeroing for stable control |
| Battery voltage level | Configures protection thresholds for 48/60/72V | Must match actual battery pack nominal voltage |
| Maximum output current | Limits peak current during acceleration | Verify before first run — too high damages motor windings |

# 7  Typical Problems and Fault Analysis

## 7.1  Vibrates Slowly or Does Not Move After Throttle

| Item | Detail |
| --- | --- |
| Symptom | Wiring correct; after gentle throttle, vehicle vibrates slowly or does not move at all |
| Meaning | Controller IS producing output — two possible causes: |
| Cause A | Motor encoder OK but phase sequence wrong. Change parameter Swap Motor Phase Lines (toggle 0↔1) until current loop feedback speed becomes POSITIVE. |
| Cause B | Motor encoder abnormal: power supply issue, wire break, poor contact, incorrect pin sequence, or encoder damage. |
| Note | Changing Swap Motor Phase Lines is only effective when the encoder is functioning correctly. |

## 7.2  Vehicle Does Not Move or Overspeeds (Runaway) After Throttle

| Check | Action Required |
| --- | --- |
| Check 1 — Phase direction | Current loop test: feedback RPM MUST be POSITIVE and satisfy: RPM = (output frequency × 60) ÷ pole pairs. If negative → swap phase lines. |
| Check 2 — Encoder signal | Motor rotates but no speed feedback → inspect B2–B8 wiring; replace encoder if wiring confirmed correct. |
| Check 3 — Angle zeroing | Resolver electrical angle offset wrong. With Check 1 satisfied, re-zero (repeat multiple times). Offset must be within ±50. |
| Check 4 — Hardware | If all above checks pass and fault persists → hardware damage; return to factory. |

## 7.3  No Response After Key-On and Throttle

| Required Condition | How to Verify |
| --- | --- |
| Direction signal present | Check that forward or reverse input is active (A11 for reverse, default is forward) |
| No brake signal active | Verify no brake lever engaged; measure A17 (low brake) and A24 (high brake) |
| Throttle output present | Measure A30: must rise from ~0.8V (idle) to ~4.2V (full throttle) |
| Supply voltage above threshold | Measure battery voltage; compare with configured under-voltage limit in software |

# 8  Fault Alarms and Remedies

The controller signals faults through the buzzer using Long (L) and Short (S) beep patterns. Each pattern repeats continuously until the fault is resolved.

Timing: Long beep ~0.8 s  |  Short beep ~0.2 s  |  Gap ~0.2 s

Figure 15 — Fault alarm beep code reference chart — all 13 fault codes with visual beep patterns

| No. | Pattern | Fault Name | Cause and Remedy |
| --- | --- | --- | --- |
| 1 | Continuous long | High Throttle Fault | Throttle held at power-on. Release throttle and restart. Persistent → check A30/A22/A29 wiring. |
| 2 | 1L + 2S | Pre-Charge Fault | Pre-charge failed. Restart. Persistent → check power supply, main fuse, BMS output voltage. |
| 3 | 1L + 3S | Overcurrent | Motor circuit short or encoder abnormality. Remove motor and restart. Persistent → return to factory. |
| 4 | 1L + 4S | Controller Overheat | Controller too hot. Stop and cool. Improve mounting ventilation. |
| 5 | 1L + 5S | Controller Overload | Output current exceeded rated for too long. Reduce load. |
| 6 | 1L + 6S | Short Circuit | Motor main circuit short or current loss. Remove motor and restart. Persistent → return to factory. |
| 7 | 1L + 9S | Battery Under-Voltage | Supply below threshold. Charge battery; check BMS; check B+/B− wiring. |
| 8 | 1L + 10S | Battery Over-Voltage | Supply above threshold. Check BMS and charger output. |
| 9 | 1L + 11S | Motor Overheat | Motor too hot. Stop and cool. Fault persists after cooling → check B1/B5 wiring. |
| 10 | 1L + 13S | Throttle Fault | Throttle signal abnormal. Check A30 is within 0–5V range throughout travel. |
| 11 | 1L + 14S | Current Sensor Fault | Output current detection abnormal. Remove motor and restart. Persistent → return to factory. |
| 12 | 1L + 15S | Encoder Error | Resolver mismatch or signal fault. Inspect all B2–B8 wiring and resolver. |
| 13 | 1L + 16S | Comm. Timeout | Communication disconnected in command-control mode. Check RS485/CAN wiring. |

## 8.1  General Fault Resolution Steps

- Note the exact alarm beep pattern and identify the fault from the table above.

- Power off completely and wait at least 15 minutes (capacitor discharge).

- Address the specific cause described for that fault code.

- Power on and verify the fault is cleared (no alarm beep).

- If fault persists after all listed checks, contact Windcon technical support or return to factory.

⚠ DANGER — Never diagnose faults while energized!

- Always cut all power and wait at least 15 minutes before inspecting wiring or components. - When returning for repair, document the exact beep pattern and operating conditions observed.

# Appendix A  —  List of Figures

| Figure 1 FDK3533C-XC Annotated Controller Photo — All Major Components Labelled | (see page) |
| --- | --- |

| Figure 2 Controller Port Distribution — Original Technical Drawing (English Labels) | (see page) |
| --- | --- |

| Figure 3 30-Pin IO Connector — Original Layout Schematic (English Labels) | (see page) |
| --- | --- |

| Figure 4 30-Pin IO Connector — Color-Coded Pin Map by Signal Type | (see page) |
| --- | --- |

| Figure 5 8-Pin Encoder Connector — Original Layout Schematic (English Labels) | (see page) |
| --- | --- |

| Figure 6 8-Pin Encoder Connector — Color-Coded SIN/COS/REF Diagram | (see page) |
| --- | --- |

| Figure 7 Power & Motor Terminal Layout — Original Technical Drawing (English Labels) | (see page) |
| --- | --- |

| Figure 8 Mechanical Installation Dimensions — Original Factory Drawing | (see page) |
| --- | --- |

| Figure 9 Typical Application Wiring Diagram — Variant A (Single 12V DC-DC) | (see page) |
| --- | --- |

| Figure 10 Typical Application Wiring Diagram — Variant B (Dual 12V DC-DC) | (see page) |
| --- | --- |

| Figure 11 System Wiring Overview — All External Connections with Color-Coded Signal Paths | (see page) |
| --- | --- |

| Figure 12 Resolver (Rotary Transformer) — Operating Principle and Signal Waveforms | (see page) |
| --- | --- |

| Figure 13 FOC / SVPWM Vector Control Loop — Complete Block Diagram | (see page) |
| --- | --- |

| Figure 14 Signal Logic Guide — Active High vs. Active Low vs. Analog Throttle | (see page) |
| --- | --- |

| Figure 15 Fault Alarm Beep Code Reference Chart — All 13 Fault Codes | (see page) |
| --- | --- |

| Figure 16 Integration Guide — Connecting Nine External Sub-Systems | (see page) |
| --- | --- |

| Figure 17 Commissioning Flowchart — Complete Initial Setup Procedure | (see page) |
| --- | --- |

# Appendix B  —  List of Abbreviations

| Abbreviation | Full Term | Meaning / Context in This Manual |
| --- | --- | --- |
| ADC | Analog-to-Digital Converter | Converts throttle voltage on A30 (0–5V) to a digital speed command inside the controller DSP |
| BMS | Battery Management System | External device managing battery charge, discharge, and cell protection; connects via CAN bus |
| CAN | Controller Area Network | ISO 11898 serial bus; connected on A2 (CAN L) and A3 (CAN H); used for instruments, BMS, VCU |
| COS | Cosine | One of two differential resolver output signals; carries rotor angle as amplitude-modulated cosine wave |
| DSP | Digital Signal Processor | High-speed processor inside controller performing FOC, PWM, and resolver calculations in real time |
| EMC | Electromagnetic Compatibility | Use shielded cables and proper grounding to ensure compliance and prevent interference |
| ESD | Electrostatic Discharge | Static electricity that can damage electronics; wear anti-static wristband when wiring |
| FOC | Field-Oriented Control | Vector control algorithm that independently controls torque and flux currents for smooth, efficient operation |
| GND | Ground | Electrical reference potential (0V). All signal inputs are referenced to B− (battery negative) |
| Id | D-axis Current | Direct-axis (flux-producing) current in the FOC rotating reference frame; controls field weakening |
| Iq | Q-axis Current | Quadrature-axis (torque-producing) current in the FOC rotating reference frame; controls torque output |
| IO | Input/Output | General term for signal pins on the 30-pin connector (Connector A) |
| IP65 | Ingress Protection 65 | Dust-tight and protected against low-pressure water jets; the controller housing protection rating |
| KTY84-150 | Temperature Sensor | Silicon PTC thermistor, -50°C to +150°C range; default motor temperature sensor (B1/B5) |
| NDA | Non-Disclosure Agreement | This document is for internal use under NDA only — do not distribute externally |
| PI | Proportional-Integral | Feedback control algorithm used in FOC speed loop and current loops to minimize steady-state error |
| PMSM | Permanent Magnet Synchronous Motor | Three-phase AC motor with permanent magnets on rotor; the motor type this controller drives |
| PWM | Pulse Width Modulation | High-frequency switching used to control average voltage/current to motor phases |
| REF | Reference / Excitation | ~10 kHz AC signal generated by DSP and supplied to resolver primary winding via B4/B8 |
| ROHS | Restriction of Hazardous Substances | EU directive; all wire harness materials must comply |
| RPM | Revolutions Per Minute | Motor speed. Formula: RPM = output frequency × 60 ÷ motor pole pairs |
| RS485 | Recommended Standard 485 | Differential serial standard; used for commissioning and command control on A8/A9 |
| SIN | Sine | One of two differential resolver output signals; carries rotor angle as amplitude-modulated sine wave |
| SVPWM | Space Vector PWM | Advanced modulation in FOC; maximizes voltage utilization and reduces harmonic distortion |
| VCU | Vehicle Control Unit | Higher-level vehicle computer sending speed/torque commands to this controller via CAN bus |

# Appendix C  —  Technical Explainer Diagrams

This appendix provides six original technical diagrams explaining how the FDK3533C-XC operates internally and how to correctly integrate it with external systems.

## C.1  Resolver (Rotary Transformer) — Operating Principle

The FDK3533C-XC uses a resolver instead of hall sensors or optical encoders. A resolver is a contactless electromagnetic angle sensor that outputs differential sine and cosine voltages whose amplitudes vary with rotor position. The controller DSP supplies a ~10 kHz excitation signal to the resolver primary winding and reads back the modulated SIN/COS outputs to calculate absolute rotor angle (θ) and speed. This gives ~0.1° resolution with no breakable parts, no magnets to demagnetize, and operation from −55°C to +155°C.

Figure 12 — Resolver operating principle — physical diagram, signal waveforms, angle formula, and advantages vs. alternatives

## C.2  FOC / SVPWM Vector Control Loop

The controller implements full closed-loop FOC using Space Vector PWM. Three-phase currents are transformed into a rotating d-q reference frame; torque current (Iq) and flux current (Id) are controlled independently by PI regulators; results are inverse-transformed back to three-phase voltages and modulated via SVPWM to drive the IGBT/MOSFET inverter. This delivers smooth, efficient, precise torque at all speeds — superior to trapezoidal or ON/OFF commutation.

Figure 13 — FOC / SVPWM vector control loop — complete block diagram from throttle to motor shaft

## C.3  Signal Logic Guide — Active High vs. Active Low vs. Analog Throttle

Most control inputs use active-low logic (triggered by connecting to GND). The high-brake input A24 uses active-high (12V). The throttle A30 is a 0–5V analog signal. Understanding these differences is critical for correct wiring and fault diagnosis.

Figure 14 — Signal logic guide — timing diagrams, wiring circuits, and throttle voltage mapping

## C.4  Fault Alarm Beep Code Reference Chart

All 13 fault codes are illustrated with visual beep-pattern bars, fault names, and remedies. This chart is a field-ready quick-reference. See Section 8 for full descriptions. Tape or laminate a copy near the installation for maintenance use.

## C.5  Integration Guide — Connecting External Systems

This nine-panel diagram shows the exact pins, wiring rules, and critical notes for connecting: CAN bus, RS485 commissioning, instrument types (pulse, single-wire, CAN), throttle, brakes (low and high), 3-speed selector, motor direction, battery voltage selection, and anti-theft motor lock.

Figure 16 — Integration guide — nine external sub-systems with pin numbers, wiring rules, and critical notes

## C.6  Commissioning Flowchart

This step-by-step flowchart covers the full initial commissioning procedure. Critical decision points are highlighted: current loop test (Step 9, feedback MUST be POSITIVE) and electrical angle zeroing (Step 10, error must be within ±50). Never skip unloaded testing or the zeroing procedure.

Figure 17 — Commissioning flowchart — complete initial setup procedure with decision points

Guilin Windcon Technology Co., Ltd.

www.windcon.com.cn  |  FDK3533C-XC User Manual V1.0 EN  |  For internal / NDA use only