WINDCON

— Leader of Motor Control —

Permanent Magnet Synchronous Motor Controller

USER MANUAL

Model: FDK3533C-XC  (Electric Tricycle Version)

Figure 1 — FDK3533C-XC annotated controller — all major components labelled

Document Information Version: V1.0 EN ¦ Based on Oct 2023 original ¦ For internal / NDA use only Guilin Windcon Technology Co., Ltd. ¦ www.windcon.com.cn

# Table of Contents

| Preface & Version History | 3 |
| --- | --- |
| 1 Safety Information | 3 |
| 2 Product Overview | 5 |
| 3 Controller Port Description | 6 |
| 3.1 Physical Layout | 6 |
| 3.2 30-Pin IO Connector (Connector A) | 7 |
| 3.3 8-Pin Encoder Connector (Connector B) | 9 |
| 3.4 Power & Motor Terminal Studs | 10 |
| 4 Controller Resource Configuration | 10 |
| 4.1 Signal Inputs | 10 |
| 4.2 Signal Output | 11 |
| 4.3 Voltage Outputs | 11 |
| 4.4 Communication Interfaces | 11 |
| 4.5 Encoder & Temperature Feedback | 12 |
| 4.6 Original Factory Wiring Diagrams | 12 |
| 5 Installation | 14 |
| 5.1 Environmental Requirements | 14 |
| 5.2 Mechanical Dimensions & Mounting | 14 |
| 5.3 Electrical Installation Safety | 15 |
| 5.4 Post-Installation Checklist | 15 |
| 6 Commissioning | 16 |
| 7 Typical Problems & Fault Analysis | 17 |
| 8 Fault Alarms & Remedies | 18 |
| Appendix A — List of Figures | 19 |
| Appendix B — List of Abbreviations | 20 |
| Appendix C — Technical Explainer Diagrams | 22 |
| C.1 Resolver Operating Principle | 22 |
| C.2 FOC / SVPWM Control Loop | 23 |
| C.3 Signal Logic Guide | 23 |
| C.4 Fault Beep Code Chart | 24 |
| C.5 Integration Guide | 24 |
| C.6 Commissioning Flowchart | 25 |

# Preface

Thank you for your support of Windcon FDK series motor controllers. This manual covers the FDK3533C-XC: safety information, mechanical and electrical installation, port descriptions, commissioning, maintenance, and integration guidance — with all original diagrams translated and annotated.

Read carefully before use and retain for future reference.

## Version History

| Date | Version | Description |
| --- | --- | --- |
| October 2023 | V1.0 | Initial Chinese release |
| (current) | V1.0 EN | Full English translation — all 17 original diagrams annotated + 6 new explainer diagrams + 3 appendices |

# 1  Safety Information

## 1.1  Safety Alert Levels

⚠ DANGER — Death or serious injury WILL result if instructions are not followed.

⚠ WARNING — Death or serious injury MAY result if instructions are not followed.

⚠ CAUTION — Minor injury or equipment damage may result if instructions are not followed.

## 1.2  Declarations

- Read and follow all safety information before installing, operating, or maintaining this product.

- Use only in environments meeting design specifications. Non-compliance voids warranty.

Windcon assumes no liability for damage caused by:

- Non-compliance with operating instructions

- Unauthorized modification or special-application use (e.g. lifting equipment)

- Improper operation, natural events, or transport damage

## 1.3  Safety Precautions

### Wiring

⚠ DANGER — Wiring

- Non-professionals STRICTLY PROHIBITED from wiring! - Never wire while power is connected — electric shock risk. - Cut ALL power; wait ≥15 min before touching — capacitor residual voltage. - Ensure reliable grounding; follow ESD procedures and wear anti-static wristband.

⚠ WARNING — Wiring

- NEVER connect power to motor output terminals U/V/W — fire risk! - Phase sequence must match exactly. Reversed polarity damages product or encoder. - Use shielded cable; ground shield at controller end only.

### Power-On

⚠ DANGER — Power-On

- Motor MUST be unloaded for all initial commissioning. - Confirm supply voltage matches controller rating before power-on. - Devices may move at power-on — maintain safe distance! - Never touch terminals, open covers, or disassemble while energized.

### Operation

⚠ DANGER — Operation

- Never touch terminals, covers, or components while running. - Never touch housing/fan/motor to test temperature — burn risk! - Non-professionals must not probe signals during operation.

⚠ WARNING — Operation

- Prevent foreign objects from falling into equipment. - Do NOT use a contactor on/off for start/stop control — causes damage.

### Maintenance & Repair

⚠ DANGER — Maintenance/Repair

- Non-professionals STRICTLY PROHIBITED! - Never work on powered equipment. Cut all power; wait ≥15 min. - After replacement: re-confirm wiring, correct parameters, run unloaded first.

# 2  Product Overview

## 2.1  Introduction

The FDK3533C-XC is a low-voltage PMSM controller designed for electric tricycles, developed by Guilin Windcon Technology Co., Ltd. It employs advanced Field-Oriented Control (FOC / SVPWM) for precise torque control across a wide speed range, delivering stable smooth driving with multiple safety protections.

## 2.2  Key Features

- Resolver encoder: High-temp, non-demagnetizing, vibration/dust/oil-proof. No hall sensors. No encoder damage from heat or water.

- Dual thermal protection in motor and controller — reduces failures and repairs significantly.

- Anti-rollback, downhill speed limiting, regenerative braking.

- Flux weakening + 3-speed mode: Adds up to 8 km/h to top speed.

- Speed: 40–55 km/h  |  Load: 1 ton  |  Gradient: 30%  |  Efficiency: up to 94%

- Soft-start, anti-gear-clash  |  CAN + RS485 communication

## 2.3  Technical Specifications

| Parameter | Specification |
| --- | --- |
| Model | FDK3533C-XC |
| Dimensions (w/o rain cover) | 205 × 165 × 71.5 mm |
| Dimensions (with rain cover) | 205 × 176.6 × 82.1 mm |
| Max Bus Current | DC 140 A |
| Max Phase Current | 350 A rms |
| Rated Operating Voltage | 48 / 60 / 72 V DC |
| Supply Voltage Range | 18 – 96 V DC |
| Control Method | SVPWM (Space Vector PWM) |
| Insulation Resistance | >=20 MΩ normal ¦ >=1 MΩ high temp/humidity |
| Protection Rating | IP65 |
| Cooling | Natural convection — install in ventilated location |
| Motor Type | PMSM (Permanent Magnet Synchronous Motor) |
| Encoder | Resolver (rotary transformer) |
| Operating Temperature | -25°C to +60°C |
| Storage Temperature | -40°C to +85°C |
| Humidity | 0–100% RH non-condensing |

# 3  Controller Port Description

## 3.1  Physical Layout

The FDK3533C-XC has four main connection areas. Figure 1 (cover) shows the annotated photo. Figure 2 shows the original top-view technical drawing.

Figure 2 — Controller port distribution — original technical drawing, English labels added

| Area | Connector | Contents |
| --- | --- | --- |
| Power & Motor | 5× stud terminals (B+, B−, U, V, W) | Battery +/− and motor phases — 25–36 mm² |
| Functional IO | 30-pin connector (Connector A) | Signals, CAN, RS485, throttle, brakes, key, speed, READY |
| Encoder Feedback | 8-pin connector (Connector B) | Resolver SIN/COS/REF (±) and motor temperature |
| Mechanical | 4× mounting holes | Ø8.5 mm, M8 bolts, 185×145 mm pattern |

## 3.2  30-Pin IO Connector (Connector A)

Connector type: Electric vehicle controller 30P harness plug. Wire colors shown are reference only.

Figure 3 — 30-Pin IO connector — original layout schematic with English labels

Figure 4 — 30-Pin IO connector — color-coded pin map by signal function

| PIN | Wire | Signal | Description |
| --- | --- | --- | --- |
| A1 | Brn/Wht | Wheel Motion | Pulse when motor rotates — connect to alarm/buzzer |
| A2 | Red/Wht | CAN L | CAN bus Low (protocol customizable) |
| A3 | Red | CAN H | CAN bus High (protocol customizable) |
| A4 | LGrn/Wht | Isolated GND | CAN/RS485 isolation GND — connect in noisy environments |
| A5 | Pnk/Wht | Isolated 5V | Isolated 5V out (max 100 mA) — reserved for Bluetooth |
| A6 | Grn/Wht | Speed Pulse/1-Wire | 8 pulses/rev (5V) for speedometer OR single-wire comm. |
| A7 | Black | Instrument GND | Ground for pulse speedometer |
| A8 | Blu/Wht | RS485 T+ | RS485 transmit + — commissioning / command control |
| A9 | Pur/Wht | RS485 T− | RS485 transmit − — commissioning / command control |
| A10 | Purple | Key Switch | B+ input to energize controller via key switch |
| A11 | Brown | Reverse | Active LOW → reverse direction |
| A12 | Orange | Mid/Low Speed | Active LOW → mid/low speed mode |
| A13 | Purple | Key SW (Alarm) | Alarm unit sends key-on to start controller |
| A14 | Lt Blue | Alarm / Lock | Active LOW → alarm active, motor locked |
| A15 | Blue | High Speed | Active LOW → high speed / flux weakening mode |
| A16 | Black | Signal GND | Common GND for active-low inputs |
| A17 | Gray | Low Brake | Active LOW → brake (brake lever switch) |
| A18 | Yellow | P-Gear | Active LOW → enter/exit parking gear mode |
| A19 | Black | Signal GND | Common GND for active-low signals |
| A20 | Org/Wht | Voltage Select | Active LOW → switch voltage profile (48/60/72V) |
| A21 | Yel/Blk | P-Gear Select | Active LOW → enable/disable P-gear feature |
| A22 | Pink | Throttle Supply + | 5V out to power throttle sensor (max 200 mA) |
| A23 | Black | Direction Select | Active LOW → swap motor phase lines |
| A24 | Gry/Wht | High Brake | Active HIGH (12V) → brake — use A25 as 12V source |
| A25 | Lt Green | 12V+ Output | 12V out for active-high switch signals (max 200 mA) |
| A26 | White | READY Signal | 5V out to READY lamp on instrument cluster |
| A27 | Blk/Wht | Signal GND | Common GND for active-low signals |
| A28 | Yel/Wht | B+ Output | Battery voltage out for key switch only (max 1 A) |
| A29 | Black | Throttle GND | Ground for throttle power supply |
| A30 | Green | Throttle Signal | Throttle analog input 0–5V |

⚠ CAUTION — 30-Pin Connector

- Specific insertion direction — reverse insertion PROHIBITED! - Harness: 0.5 mm² ROHS, pull force ≥6 kgf, IP66 housing, 80°C 300V rated.

## 3.3  8-Pin Encoder Feedback Connector (Connector B)

Carries resolver signals and motor temperature. Use shielded cable; ground shield at controller end only.

Figure 5 — 8-Pin encoder connector — original layout schematic with English labels

Figure 6 — 8-Pin encoder connector — color-coded SIN/COS/REF + temperature diagram

| PIN | Wire | Signal | Description |
| --- | --- | --- | --- |
| B1 | Red/Wht | Motor Temp (+) | Motor temp sensor positive — default KTY84-150 |
| B2 | Blue | COS+ | Resolver cosine positive |
| B3 | Yellow | SIN+ | Resolver sine positive |
| B4 | Red | REF+ | Resolver excitation positive (~10 kHz AC) |
| B5 | Blk/Wht | Motor Temp (−) | Motor temp sensor negative — default KTY84-150 |
| B6 | White | COS− | Resolver cosine negative |
| B7 | Green | SIN− | Resolver sine negative |
| B8 | Black | REF− | Resolver excitation negative |

⚠ CAUTION — 8-Pin Encoder Connector

- Specific insertion direction — reverse insertion PROHIBITED! - Non-KTY84-150 sensors require Windcon engineering adaptation. - Use shielded cable. Ground shield at controller end only.

## 3.4  Power & Motor Terminal Studs

Figure 7 — Power and motor terminal layout — original technical drawing with English labels

| Pos. | Label | Function | Cable | Terminal | Tool |
| --- | --- | --- | --- | --- | --- |
| 1 | U | Motor Phase U | 25–36 mm² | RNB 38-6 | ZTGYQK-70 |
| 2 | B− | Battery Negative | 25–36 mm² | RNB 38-6 | ZTGYQK-70 |
| 3 | V | Motor Phase V | 25–36 mm² | RNB 38-6 | ZTGYQK-70 |
| 4 | B+ | Battery Positive | 25–36 mm² | RNB 38-6 | ZTGYQK-70 |
| 5 | W | Motor Phase W | 25–36 mm² | RNB 38-6 | ZTGYQK-70 |

⚠ WARNING — Power Terminals

- NEVER reverse B+ and B−! - Wiring order left→right: U → B− → V → B+ → W - 25–36 mm² tinned copper only. Crimp with ZTGYQK-70.

# 4  Controller Resource Configuration

## 4.1  Signal Inputs

| PIN | Signal | Trigger | Input Range | Max V |
| --- | --- | --- | --- | --- |
| A11 | Reverse | Active Low | 0–0.2V | 24V |
| A12 | Mid/Low speed | Active Low | 0–0.2V | 24V |
| A14 | Alarm/Lock | Active Low | 0–0.2V | 24V |
| A15 | High speed | Active Low | 0–0.2V | 24V |
| A17 | Low brake | Active Low | 0–0.2V | 24V |
| A18 | P-gear | Active Low | 0–0.2V | 24V |
| A20 | Voltage select | Active Low | 0–0.2V | 24V |
| A21 | P-gear select | Active Low | 0–0.2V | 24V |
| A23 | Direction select | Active Low | 0–0.2V | 24V |
| A24 | High brake | Active High (12V) | 10–18V | 24V |
| A30 | Throttle (analog) | 0–5V nominal | 0–10V | 12V |

⚠ CAUTION — Inputs

- Operate within typical voltage range. Exceeding limits may damage controller. - Inputs are positive-voltage triggered. Reference is B−. Never inject negative voltage.

## 4.2  Signal Output

| PIN | Signal | Voltage | Max Current |
| --- | --- | --- | --- |
| A26 | READY | 5V ±2% | 50 mA |

## 4.3  Voltage Outputs

| PIN | Signal | Voltage | Max mA | Permitted Use |
| --- | --- | --- | --- | --- |
| A5 | Isolated 5V | 5V ±2% | 100 | Bluetooth module only |
| A22 | Throttle + | 5V ±2% | 200 | Throttle / hall sensor |
| A25 | 12V+ output | 12V ±2% | 200 | Active-high switch signals only |
| A28 | B+ output | Battery V | 1000 | Key switch signal only |

⚠ CAUTION — Outputs

- ONLY for: throttle, Bluetooth, brake, key switch, active-high IO. - Do NOT power lights or instruments — overcurrent damages controller. - Never inject external voltage into output pins.

## 4.4  Communication Interfaces

| Interface | Pins | Type | Notes |
| --- | --- | --- | --- |
| CAN Bus | A2 (L), A3 (H) | Isolated CAN | Built-in 120Ω; add 120Ω at each bus end; protocol customized. |
| CAN Isolation GND | A4 | ISO GND | Connect to chassis in high-interference environments. |
| RS485 | A8 (T+), A9 (T−) | Isolated RS485 | Built-in 120Ω; for commissioning and command control. |
| Pulse Speed | A6, A7 | 5V pulse | 8 pulses/rev; A7=instrument GND. Default mode for A6. |
| Single-Wire | A6 only | 1-wire | Firmware mode; protocol requires Windcon customization. |
| CAN Instrument | A2, A3 | Isolated CAN | Shared bus with BMS/VCU; protocol customized separately. |

## 4.5  Encoder and Temperature Feedback

| Pins | Signal | Notes |
| --- | --- | --- |
| B2/B6 (COS+/−) | Cosine differential | Connect to motor resolver cosine winding |
| B3/B7 (SIN+/−) | Sine differential | Connect to motor resolver sine winding |
| B4/B8 (REF+/−) | Excitation ~10kHz | Supplied by controller DSP to resolver primary |
| B1/B5 (Temp+/−) | Motor temperature | Default KTY84-150; other types need Windcon adaptation |

## 4.6  Original Factory Wiring Diagrams

The two original factory schematics below show all connections. Variant A uses a single-output DC-DC; Variant B uses a dual-output DC-DC with 12V+ and 12V−.

Figure 9 — Typical application wiring diagram — Variant A: single DC-DC (12V only)

Figure 10 — Typical application wiring diagram — Variant B: DC-DC with 12V+ and 12V−

Figure 11 — System wiring overview — all external connections with color-coded signal paths

# 5  Installation

## 5.1  Environmental Requirements

- No corrosive gas/liquid, flammable/explosive, water/steam/oil/dust, or high-heat environments

- No severe electromagnetic interference or severe grid fluctuations

- Vibration environments: contact Windcon before installation

## 5.2  Mechanical Dimensions and Mounting

Figure 8 — Mechanical installation dimensions — original factory drawing

| Dimension | Value |
| --- | --- |
| Overall length | 205.0 mm |
| Width (w/o rain cover) | 165.0 mm |
| Width (with rain cover) | 176.6 mm |
| Height (w/o rain cover) | 71.5 mm |
| Height (with rain cover) | 82.1 mm |
| Hole span (length × width) | 185.0 × 145.0 mm (centre-to-centre) |
| Hole diameter | 4× Ø8.5 mm — M8 bolts |
| Base-to-datum offset | 22.0 mm |

## 5.3  Electrical Installation Safety

- All wiring by trained, authorized engineers only.

- Verify controller voltage rating before power-on; factory dielectric test done — do not repeat.

- Both controller and motor must be reliably grounded.

- Connect to supply via circuit breaker or fuse (overcurrent protection).

- NEVER connect power supply to motor terminals U, V, or W.

## 5.4  Post-Installation Inspection Checklist

| Inspection Item | Pass Criteria |
| --- | --- |
| Controller condition | No damage; securely mounted; heatsink fins clear |
| Cable condition | No damage; power cables separated from signal/encoder cables |
| Connectors | All fully mated; no tension; sufficient slack |
| Power polarity | B+/B− correct; power NOT connected to signal or encoder ports |
| Motor phases | U/V/W correct sequence; no stripped insulation |
| Grounding | Correctly and firmly connected |
| Terminal voltages | All within typical range; no over-voltage |
| Encoder | Resolver lines correctly matched; shielded cable used; shield grounded at controller end |

# 6  Commissioning

Software: 'Windcon Debug Assistant'. See its user manual. If version mismatch error appears — contact Windcon for startup config file.

## 6.1  Commissioning Sequence

| Step | Action | Critical Notes |
| --- | --- | --- |
| 1 | Mount in ventilated location | Heatsink must have airflow clearance |
| 2 | Wire B+, B−, U, V, W power terminals | Verify polarity; follow phase order; crimp properly |
| 3 | Wire encoder B2–B8 and temp B1/B5 | Shielded cable; check 8-pin orientation before mating |
| 4 | Wire all 30-pin IO signals | Key switch, throttle (A22/A29/A30), brakes, direction, speed |
| 5 | Set motor to NO LOAD | REQUIRED for all commissioning and angle zeroing |
| 6 | Connect PC via RS485 USB adapter | Open Windcon Debug Assistant |
| 7 | Power ON controller | Listen for alarm beep codes (see Section 8) |
| 8 | Verify PC communication | Read parameters; confirm voltage matches battery |
| 9 | Run current loop test | Feedback RPM MUST be POSITIVE → if negative, swap phase lines (0↔1) |
| 10 | Electrical angle zeroing | Repeat 3+ times; offset error must be <±50 |
| 11 | Test throttle response | Slow ramp-up; verify smooth control, no oscillation |
| 12 | Test brakes and direction | All safety inputs must function before load testing |
| 13 | Gradual load testing | Monitor temperatures during extended runs |

## 6.2  Key Parameters

| Parameter | Function | Action Required |
| --- | --- | --- |
| Motor rated current | Sets motor protection current limit | Must not exceed controller rated value |
| Motor pole pairs | Used in: RPM = freq×60÷pole pairs | Wrong value causes speed errors and instability |
| Swap phase lines (0/1) | Reverses motor direction | Set so current-loop feedback speed is POSITIVE |
| Angle zero offset | Rotor reference for FOC | Must be within ±50 after repeated zeroing |
| Battery voltage level | Sets 48/60/72V protection thresholds | Must match actual battery pack voltage |
| Max output current | Limits peak current | Verify before first run — too high damages motor |

# 7  Typical Problems and Fault Analysis

## 7.1  Vibrates Slowly or Does Not Move After Throttle

| Item | Detail |
| --- | --- |
| Symptom | Wiring correct; after gentle throttle, vehicle vibrates slowly or does not move |
| Cause A | Motor phase sequence wrong → toggle Swap Phase Lines (0↔1) until current-loop feedback speed is POSITIVE |
| Cause B | Encoder abnormal: power issue, wire break, poor contact, wrong sequence, or encoder damage |
| Note | Swap Phase Lines only works when encoder is functioning correctly |

## 7.2  Does Not Move or Overspeeds (Runaway) After Throttle

| Check | Action |
| --- | --- |
| 1 — Phase direction | Current loop: feedback RPM MUST be POSITIVE and satisfy RPM = freq×60÷pole pairs |
| 2 — Encoder signal | Motor rotates but no speed feedback → inspect B2–B8; replace encoder if wiring is correct |
| 3 — Angle zeroing | Re-zero with Check 1 satisfied; repeat; offset must be within ±50 |
| 4 — Hardware | All above pass but fault persists → return to factory |

## 7.3  No Response After Key-On and Throttle

| Required Condition | How to Verify |
| --- | --- |
| Direction signal present | Check forward or reverse input is active (A11 for reverse) |
| No brake active | Verify no brake lever pressed; measure A17 and A24 |
| Throttle has output | Measure A30: must rise ~0.8V (idle) to ~4.2V (full) |
| Supply above threshold | Measure battery voltage vs configured under-voltage limit |

# 8  Fault Alarms and Remedies

Faults are signalled via buzzer using Long (L) + Short (S) beep patterns repeating until resolved.

Timing: Long ≈0.8 s  |  Short ≈0.2 s  |  Gap ≈0.2 s

Figure 15 — Fault alarm beep code reference chart — all 13 codes with visual patterns and remedies

| No. | Pattern | Fault | Remedy |
| --- | --- | --- | --- |
| 1 | Continuous long | High Throttle | Throttle held at power-on. Release and restart. Persistent → check A30/A22/A29. |
| 2 | 1L+2S | Pre-Charge | Pre-charge failed. Restart. Persistent → check supply, fuse, BMS. |
| 3 | 1L+3S | Overcurrent | Motor circuit short or encoder fault. Remove motor, restart. Persistent → factory. |
| 4 | 1L+4S | Controller Overheat | Too hot. Stop and cool. Improve mounting ventilation. |
| 5 | 1L+5S | Controller Overload | Current exceeded rated too long. Reduce load. |
| 6 | 1L+6S | Short Circuit | U/V/W short or current loss. Remove motor, restart. Persistent → factory. |
| 7 | 1L+9S | Battery Under-Voltage | Supply below threshold. Charge battery; check BMS; check B+/B−. |
| 8 | 1L+10S | Battery Over-Voltage | Supply above threshold. Check BMS and charger. |
| 9 | 1L+11S | Motor Overheat | Motor too hot. Stop and cool. Persists after cooling → check B1/B5. |
| 10 | 1L+13S | Throttle Fault | Check A30 is 0–5V throughout throttle travel. |
| 11 | 1L+14S | Current Sensor | Abnormal detection. Remove motor, restart. Persistent → factory. |
| 12 | 1L+15S | Encoder Error | Resolver mismatch or signal fault. Inspect all B2–B8 wiring and resolver. |
| 13 | 1L+16S | Comm. Timeout | Communication lost in command mode. Check RS485/CAN wiring. |

## 8.1  Fault Resolution Steps

- Note the exact beep pattern and identify the fault from the table above.

- Power off completely and wait at least 15 minutes.

- Address the specific cause for that fault code.

- Power on and verify fault is cleared.

- If persists, contact Windcon or return to factory.

⚠ DANGER — Never diagnose faults while energized!

- Cut all power; wait ≥15 min before inspecting wiring. - When returning for repair, document the beep pattern and operating conditions.

# Appendix A  —  List of Figures

| Figure 1 FDK3533C-XC Annotated Controller Photo — All Major Components Labelled | see page |
| --- | --- |
| Figure 2 Controller Port Distribution — Original Technical Drawing (English Labels) | see page |
| Figure 3 30-Pin IO Connector — Original Layout Schematic (English Labels) | see page |
| Figure 4 30-Pin IO Connector — Color-Coded Pin Map by Signal Function | see page |
| Figure 5 8-Pin Encoder Connector — Original Layout Schematic (English Labels) | see page |
| Figure 6 8-Pin Encoder Connector — Color-Coded SIN/COS/REF Diagram | see page |
| Figure 7 Power & Motor Terminal Layout — Original Technical Drawing (English Labels) | see page |
| Figure 8 Mechanical Installation Dimensions — Original Factory Drawing | see page |
| Figure 9 Typical Application Wiring Diagram — Variant A (Single 12V DC-DC) | see page |
| Figure 10 Typical Application Wiring Diagram — Variant B (Dual 12V DC-DC) | see page |
| Figure 11 System Wiring Overview — All External Connections, Color-Coded | see page |
| Figure 12 Resolver (Rotary Transformer) — Operating Principle and Signal Waveforms | see page |
| Figure 13 FOC / SVPWM Vector Control Loop — Complete Block Diagram | see page |
| Figure 14 Signal Logic Guide — Active High vs. Active Low vs. Analog Throttle | see page |
| Figure 15 Fault Alarm Beep Code Reference Chart — All 13 Fault Codes | see page |
| Figure 16 Integration Guide — Connecting Nine External Sub-Systems | see page |
| Figure 17 Commissioning Flowchart — Complete Initial Setup Procedure | see page |

# Appendix B  —  List of Abbreviations

| Abbreviation | Full Term | Meaning / Context |
| --- | --- | --- |
| ADC | Analog-to-Digital Converter | Converts throttle voltage on A30 (0–5V) to digital speed command inside the DSP |
| BMS | Battery Management System | Manages battery charge/discharge/protection; connects via CAN (A2/A3) |
| CAN | Controller Area Network | ISO 11898 serial bus; connected on A2 (CAN L) and A3 (CAN H) |
| COS | Cosine | Resolver differential output signal; amplitude encodes rotor angle as cosine |
| DSP | Digital Signal Processor | High-speed processor inside controller for FOC, PWM, and resolver calculations |
| EMC | Electromagnetic Compatibility | Use shielded cables and proper grounding to ensure compliance |
| ESD | Electrostatic Discharge | Static electricity that can damage electronics; wear anti-static wristband when wiring |
| FOC | Field-Oriented Control | Algorithm controlling torque (Iq) and flux (Id) currents independently for smooth operation |
| GND | Ground | 0V electrical reference. All signal inputs reference to B− (battery negative) |
| Id | D-axis Current | Direct (flux-producing) current in FOC d-q frame; controls field weakening |
| Iq | Q-axis Current | Quadrature (torque-producing) current in FOC d-q frame; controls motor torque |
| IO | Input/Output | Signal pins on the 30-pin connector (Connector A) |
| IP65 | Ingress Protection 65 | Dust-tight; protected against water jets; the controller housing rating |
| KTY84-150 | Temp. Sensor Type | Silicon PTC thermistor, −50°C to +150°C; default motor temp sensor (B1/B5) |
| NDA | Non-Disclosure Agreement | This document is for internal NDA use only — do not distribute externally |
| PI | Proportional-Integral | Feedback control in FOC speed and current loops to eliminate steady-state error |
| PMSM | Perm. Magnet Sync. Motor | Three-phase AC motor with permanent magnets on rotor; what this controller drives |
| PWM | Pulse Width Modulation | High-freq switching to control average voltage/current delivered to motor phases |
| REF | Reference / Excitation | ~10 kHz AC generated by DSP and fed to resolver primary winding (B4/B8) |
| ROHS | Restriction of Hazardous Substances | EU directive; all wire harness materials must comply |
| RPM | Revolutions Per Minute | Motor speed. Formula: RPM = output frequency × 60 ÷ motor pole pairs |
| RS485 | Recommended Standard 485 | Differential serial standard; commissioning and command control (A8/A9) |
| SIN | Sine | Resolver differential output; amplitude encodes rotor angle as sine waveform |
| SVPWM | Space Vector PWM | Advanced modulation in FOC; maximizes voltage utilization, reduces harmonics |
| VCU | Vehicle Control Unit | Higher-level vehicle computer sending speed/torque commands via CAN bus |

# Appendix C  —  Technical Explainer Diagrams

Six original diagrams explaining internal operation and external integration of the FDK3533C-XC.

## C.1  Resolver (Rotary Transformer) — Operating Principle

The FDK3533C-XC uses a resolver instead of hall sensors. A resolver is a contactless electromagnetic angle sensor: the controller DSP supplies a ~10 kHz excitation signal (REF+/REF−) to the primary winding; the rotor modulates this into differential SIN and COS outputs whose amplitudes vary with rotor position. The DSP calculates absolute angle (θ = arctan(SIN/COS)) and speed (Δθ/Δt) at ~0.1° resolution — with no breakable parts, no demagnetization risk, and operation from −55°C to +155°C.

Figure 12 — Resolver operating principle — physical diagram, signal waveforms, angle formula, and advantages

## C.2  FOC / SVPWM Vector Control Loop

Full closed-loop Field-Oriented Control: three-phase currents are transformed into a rotating d-q frame; PI regulators independently control torque current (Iq) and flux current (Id); results are inverse-Park-transformed back to three-phase voltage vectors and modulated via Space Vector PWM to drive the three-phase inverter. This delivers smooth, efficient, precise torque at all speeds — far superior to trapezoidal or ON/OFF commutation.

Figure 13 — FOC / SVPWM vector control loop — complete block diagram from throttle input to motor shaft

## C.3  Signal Logic Guide — Active High vs. Active Low vs. Analog Throttle

Most control inputs use active-low logic (triggered by pulling to GND). The high-brake A24 uses active-high (12V). The throttle A30 is a 0–5V analog signal powered from A22. Understanding these differences is critical for correct wiring and fault diagnosis.

Figure 14 — Signal logic guide — timing diagrams, typical wiring circuits, and throttle voltage-speed mapping

## C.4  Fault Alarm Beep Code Reference Chart

Visual chart of all 13 fault codes with beep-pattern bar graphics. Long beep ~0.8 s, short beep ~0.2 s. Print and laminate for field maintenance use. Full descriptions and remedies are in Section 8.

## C.5  Integration Guide — Connecting External Systems

Nine-panel diagram showing exact pins, wiring rules, and critical notes for: CAN bus, RS485, instrument types (pulse/single-wire/CAN), throttle, brakes (low/high), 3-speed selector, motor direction, battery voltage selection, and anti-theft lock.

Figure 16 — Integration guide — nine external sub-systems with pin numbers, wiring rules, and critical notes

## C.6  Commissioning Flowchart

Step-by-step flowchart for full initial commissioning. Critical decision points: Step 9 (current-loop feedback MUST be POSITIVE), Step 10 (angle zeroing offset MUST be within ±50). Never skip unloaded testing or angle zeroing.

Figure 17 — Commissioning flowchart — complete initial setup procedure with decision points

Guilin Windcon Technology Co., Ltd.

www.windcon.com.cn  |  FDK3533C-XC User Manual V1.0 EN  |  For internal / NDA use only